ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL THREE | PO: PROPULSION |
| --- | --- |

PO/EO:	416.05

ENABLING OBJECTIVE:	Explain supercharging and turbocharging.

REFERENCE(S):	A.	A-CR-CCP-268/PH-001 Level Three Course Training Plan

Chapter 4, Pages 115-116.

B.	A-CR-CCP-268/PT-001 Level Three Handbook

Chapter 9, Section 5-6, Articles 35-40, Pages 12-13.

C.	A-CR-CCP-263/PT-001 From the Ground Up

Pages 64-65.

SUPPLEMENTARY REF(S):	N/A

TRAINING AID(S):		A.	OHP

B.	OHP Slides

C.	Diagrams (if available)

LEARNING AID(S):		A.	A-CR-CCP-268/PT-001 Level Three Handbook

TEACHING POINT(S):	A.	Principle

B.	Supercharging

C.	Turbo Charging

METHOD OF INSTRUCTION:	Lecture

TIME:		1 x 35-minutes

ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL THREE | PO: PROPULSION |
| --- | --- |

PO/EO:	416.05

ENABLING OBJECTIVE:	Explain supercharging and turbocharging.

REVIEW

PO/EO:	416.04b

ENABLING OBJECTIVE:	Explain carburetor systems.

1.	Review weaknesses of the last EOs performance check; and/or

2.	Ask the following review questions:

a.	What are the main components of a carburetor?

Answer:	Air passage and choke valve, throttle valve, float chamber and float mechanism, and main jet.

b.	What are the dangers of carburetor icing?

Answer:	Carburetor ice can cause a loss of power or complete engine failure.

INTRODUCTION

WHAT:	In this lesson you will learn about supercharging and turbocharging.

WHY:	Supercharging and turbocharging are terms that are often heard in relation to engines, but are not always understood.   Both of these methods for increasing engine performance are used in aircraft engines.

WHERE:	You can apply this knowledge when visiting an aerodrome for a facilities tour, at an air-show, during glider familiarization, or even while attending summer training.

| TIME | BODY | NOTES |
| --- | --- | --- |

STAGE 1	SUPERCHARGING

5

MINs

1.	General:

a.	An engine designed to operate at sea level atmospheric pressure is 		called a normally aspirated engine.

b.	As we have already learned, as altitude increases, the density of			the air decreases.  As the air becomes less and less dense with 			altitude, the engine produces less and less power.  This is indicated 		by the decreasing rate of climb and eventually the total inability to 			climb any higher.

c.	These problems can be overcome by supercharging or				turbocharging the engine.  These devices compress the air so that 			it is denser and the engine can produce more power.

2.	Supercharging:

OHP # 1

a.	The supercharger is an internally driven compressor, powered 			directly from the engine. It compresses the fuel/air mixture after it 			leaves the carburetor. This called forced induction.

b.	When forced induction is used at high altitudes to make up the 			deficiency in pressure due to the lower density of the air – and 			hence maintain sea level power – it is called supercharging.

c.	Forced induction may also be used to increase the power at low 			altitudes. In this case the pressure over and above the sea level 			atmospheric pressure that is forced into the manifold is called 			boost.

CONFIRMATION STAGE 1

1.	What is a supercharger?

Answer:	The supercharger is an internally driven compressor,					powered directly from the engine.  It compresses the fuel/air 				mixture after it leaves the carburetor.

2.	Why do we use superchargers?

Answer:	Superchargers are used at high altitudes to make up the 				deficiency in pressure due to the lower density of the air.	  			They are also used at lower altitudes to boost the engine.

STAGE 2 	TURBOCHARGING

15

MINs

1.	Turbocharging:

OHP # 2

a.	In many modern aircraft, the job of supplying the engine with dense			air when the plane is operating in the thin air of high altitudes is			accomplished by a turbocharger.

b.	A turbocharger is powered by the energy of the exhaust gases 			directed through a turbine wheel, or impeller, and turning this wheel 		at high rpm. This turbine wheel is mounted on a shaft on which an 			air compressor is also mounted.

c.	As the exhaust gases are directed over the turbine, the compressor			will turn faster and the air supplied to the engine by the compressor 		will be denser, allowing the engine to produce more power.

d.	The great advantage of turbocharging is the increase performance 			at high altitude; including, better climb performance, faster cruise at			high altitude, and better take-off performance at high density, high 			altitude airports.

2.	Compressors/Chargers:

a.	Superchargers and turbochargers do not replace normal engine 			systems - they enhance the engine capacity to produce power.

b.	Because superchargers are powered directly from the engine, they			tax the engine to a greater degree than a turbocharger.

c.	Because turbochargers are powered by the hot exhaust gases			already used by the engine and redirected through it, they are more 		energy efficient.

CONFIRMATION STAGE 2

1.	How does a turbocharger work?

Answer:	A turbocharger is powered by the energy of the exhaust 				gases.  As the exhaust gases are directed over the turbine, 				the compressor will turn faster and the air supplied to the 				engine by the compressor will be denser, allowing the 				engine to produce more power.

2.	What is the major difference between a supercharger and a			turbocharger?

Answer:	Because superchargers are powered directly from the 				engine, they tax the engine to a greater degree then 					turbocharger.  Therefore, since turbochargers are powered 				by the hot exhaust gases already used by the engine and 				redirected through it, they are more energy efficient.

| 28 MINs | PERFORMANCE CHECK |
| --- | --- |

Test Details - 	Each cadet will have to explain superchargers and turbochargers.

1.	Test knowledge with OHPs.

| 33 MINs | CONCLUSION |
| --- | --- |

SUMMARY:	A.	In this lesson you have learned about superchargers and turbochargers.

RE-MOTIVATION:	A.	Comment on student performance. (Identify strengths and points that

require improvement)

B.	You can apply this knowledge when visiting an aerodrome for a facilities		tour, at an air-show, during glider familiarization, or even while attending 	summer training.

C.	This is the last class of PO 416. Your next period will be your Propulsion test.