| PROFICIENCY LEVEL THREE ROYAL CANADIAN AIR CADETS MASTER LESSON PLAN | PO: RADIO PROCEDURES |
| --- | --- |

PO/EO:	418.02a

ENABLING OBJECTIVE:	Identify correct calling and replying radio procedures.

REFERENCE(S):	A.	A-CR-CCP-268/PH-001 Level Three Course Training Plan

Chapter 4, Pages 135-138.

B.	A-CR-CCP-268/PT-001 Level Three Handbook

Chapter 11, Section 3, Articles 7-14, Pages 6-8.

C.	RIC-21 Guide for the radiotelephone operators restricted certificate (aeronautical).

D.	A-CR-CCP-263/PT-001 From the Ground Up

Pages 205-214.

SUPPLEMENTARY REF(S):	N/A

TRAINING AID(S):		A.	OHP

B.	OHP slides

LEARNING AID(S):		A.	A-CR-CCP-268/PT-001 Level Three Handbook

B.	A handout of key phrases

TEACHING POINT(S):	A. 	Calling procedures

METHOD OF INSTRUCTION:	Lecture and practice

TIME:		1 x 35-minutes

| PROFICIENCY LEVEL THREE ROYAL CANADIAN AIR CADETS MASTER LESSON PLAN | PO: RADIO PROCEDURES |
| --- | --- |

PO/EO:	418.02a

ENABLING OBJECTIVE:	Identify correct calling and replying radio procedures.

REVIEW

PO/EO:	418.01b

ENABLING OBJECTIVE:		Identify radio procedures.

1.	Review weaknesses of the last EOs performance check; and/or

2.	Ask the following review question:

a.	What are the four parts radiotelephone contacts usually consist of?

Answer:	The call up, the reply, the message and the acknowledgement.

INTRODUCTION

WHAT:	In this lesson you will learn about calling procedures when operating a radio.

WHY:	It is important to know how to call someone on the radio properly so that you can avoid confusion due to improper procedure and so that a standard of radio communications can be maintained.

WHERE:	You will apply this knowledge any time you use a radio and have to communicate with others.

| TIME | BODY | NOTES |
| --- | --- | --- |

STAGE 1	CALLING PROCEDURES

5

MINs

1.	General:

a.	A call sign for an aircraft consists of five letters and the type of 			aircraft. The first part of the call sign is the type of aircraft, ie, 			Cessna 143. They all begin with C, This C is followed by a dash 			and then most aircraft have an F or G. The F or G are then followed			by three other letters which are assigned as part of the planes 			license. The C is never spoken.

b.	Before transmitting, you will listen for a period long enough to 			satisfy yourself that you will not cause harmful interference to 			transmissions already in progress. If such interference seems likely,			you should wait for the first break in the transmission. The identifier 			of the station being called is always spoken first, followed by the 			words “THIS IS” and your station’s identifier.

c.	Example: CESSNA ONE SEVEN TWO – FOXTROT 					ALPHA ROMEO QUEBEC THIS IS – FOXTROT ZULU 				UNIFORM YANKEE.

2.	Single Station Call:

OHP #1

Can be used as handout for the cadets

a.	If you wish to establish communication with a specific station, the 			following items will be transmitted in the order indicated:

OHP #2

(i)	The call sign of the station called (not more than three times,				once if radio conditions are good).

(ii)	The words “THIS IS”.

(iii)	The call sign of your station.

(iv)	The frequency on which you are transmitting.

(v)	The invitation to reply (“OVER”).

b.	Example:

TORONTO TOWER THIS IS CESSNA ONE EIGHT FIVE –

FOXTROT ALPHA DELTA TANGO OVER.

OHP #3

3.	General Call:

a.	If you wish to establish communication with any station within range			or in a certain area, the call should be made to “ALL STATIONS” 			using the same procedure as a single station call.

b.	Example:

ALL THE STATIONS THIS IS TORONTO AIR RADIO.

4.	Multiple Station Call:

OHP #4

a.	If more than one station is to be called simultaneously, the 				identifiers may be transmitted in any convenient sequence followed			by the words “THIS IS” and your call sign. As a general rule, 				operators replying to a multiple station call should answer in the 			order in which they have been called.

b.	CESSNA FOXTROT NOVEMBER INDIA  - INDIA PIPER

FOXTROT X-RAY QUEBEC - QUEBEC PIPER GOLF LIMA LIMA

DELTA THIS IS TORONTO TOWER.

CONFIRMATION STAGE 1

15

MINs

1.	What are the five things that should be transmitted in order when making a		single station call?

Answer:	a.	The call sign of the station called (not more than three					times,											b.	Once if radio conditions are good), The words “THIS 					IS”, 											c.	The call sign of your station, 							d.	The frequency on which you are transmitting and 				e.	The invitation to reply (“OVER”).

STAGE 2	REPLYING PROCEDURES

OHP #5

1.	Replying:

a.	Operators hearing a call directed to their station will reply as soon 			as possible and advise the calling station to proceed with message 			with the words “GO AHEAD”, or not to proceed with the message 			with the words “STAND BY”, followed by the anticipated number of 			minutes of delay.

b.	Example:

PIPER FOXTROT X-RAY QUEBEC QUEBEC THIS IS TORONTO

TOWER, GO AHEAD, OVER

PIPER FOXTROT X-RAY QUEBEC QUEBEC, THIS IS TORONTO 		TOWER STAND BY TWO MINUTES OVER.

c.	When operators of a station hear a call but are not sure of the 			identity of the calling station, they should reply immediately using 			the words “STATION CALLING”, their station’s identification, and 			the words “SAY AGAIN”, and “OVER”.

d.	Example:

STATION CALLING CESSNA FOXTROT NOVEMBER JULIET

INDIA SAY AGAIN OVER.

# CONFIRMATION STAGE 2

1.	What words should an operator reply with when receiving a call?

Answer:	Go ahead if they want the party to proceed, or stand by 				followed by the appropriate number of minutes of the delay if 			they want the party to hold.

| 28 MINs | PERFORMANCE CHECK |
| --- | --- |

Test Details – 	Each cadet will be checked independently and will be required to answer 				questions on the subject matter.

1.	Test the cadets’ using Handout #1 on calling procedures.

| 33 MINs | CONCLUSION |
| --- | --- |

SUMMARY:   	A.	In this lesson you have learned some different calling procedures.

RE-MOTIVATION:	A.	Comment on student performance. (Identify strengths and points that

require improvement)

B.	Knowledge of proper calling procedures allows a cadet to correctly use radio equipment on flying scholarship training.

C.	Your next lesson will be 418.02b, identify correct calling and radio procedures.