ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL FOUR | PO: NAVIGATION |
| --- | --- |

PO/EO:	417.03b

ENABLING OBJECTIVE:	Identify navigation terms and units of distance and speed.

REFERENCE(S):	A.	A-CR-CCP-269/PH-001 Level Four Course Training Plan

Chapter 4, Pages 85-89.

B.	A-CR-CCP-269/PT-001 Level Four Handbook

Chapter 9, Section 5, Pages 5-6.

C.	A-CR-CCP-263/PT-001 From the Ground Up

Pages 172-173 and 183.

SUPPLEMENTARY REF(S):	N/A

TRAINING AID(S):		A.	OHP

B.	OHP Slides

LEARNING AID(S):		A.	A-CR-CCP-269/PT-001 Level Four Handbook

B.	Paper, pencil and eraser

TEACHING POINT(S):	A.	Units of distance and speed

METHOD OF INSTRUCTION:	Lecture

TIME:		2 of 2 x 35-minutes

ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL FOUR | PO: NAVIGATION |
| --- | --- |

PO/EO:	417.03b

ENABLING OBJECTIVE:	Identify navigation terms and units of distance and speed.

REVIEW

PO/EO:	417.03a

ENABLING OBJECTIVE:	Identify navigation terms and units of distance and speed.

1.	Review weaknesses of the last EOs performance check; and/or

2.	Ask the following review questions:

- a.	What is groundspeed?

- Answer:	Groundspeed is the speed of the aircraft relative to the ground.

- b.	What is heading?

Answer:	The heading of an aircraft is the angle between the longitudinal axis of the 				aircraft and a meridian.  In other words, it is the direction the nose of the 				aircraft is pointing.  It can be measured from a true meridian, a magnetic 				meridian or from a compass needle.

INTRODUCTION

WHAT:	In this lesson you will learn about units of distance and speed.

WHY:	It is necessary to be able to use units of distance and speed in order to plot a flight plan.

WHERE:	You will have an opportunity to use this knowledge during the last Navigation class, where you will plot a flight plan.

| TIME | BODY | NOTES |
| --- | --- | --- |

STAGE 1	UNITS AND CONVERSIONS

5

MINs

1.	Units of Distance and Speed:

OHP # 1

a.	A Statute Mile is a distance of 5,280 feet.

b.	A Nautical Mile (6,080 feet) is the average length of one minute 			of latitude.

c.	A Kilometre is a distance of 1,000 meters.

OHP # 2

d.	A Knot is a speed of one nautical mile per hour.

e.	Mph is one statute mile per hour and Kmh is one kilometer per 			hour.

f.	The abbreviation for nautical miles is N. miles or N.M. - for statute			miles it is, S. miles or S.M. The abbreviation for kilometers is km 			and for knots is kts.

2.	Conversions:

a.	Speed

(i)	To convert knots to mph, multiply knots by 1.15;

(ii)	To convert mph to knots, divide mph by 1.15;

(iii)	To convert kilometers per hour to knots, multiply by 0.54;

(iv)	To convert km/h to mph, multiply by 0.62; and

(v)	A scale of nautical miles (based on the scale of the chart at				mid latitude) is printed on all aeronautical charts. Practically				all flight computers have statute mile-nautical mile					conversion indexes printed on the outer scale.

This is best taught using

a chalk board

or marker

board so that the steps are covered one

at a time.

b.	Hours and Minutes

(i)	To convert minutes to hours, divide by 60 (60 min = I hr).

Example:	30 min equals 30 divided by 60 = 0.5 hrs.

(ii)	To convert hours to minutes, multiply by 60.

Example: 	0.75 hrs = 0.75 x 60 = 45 min.

c.	Time in Flight

To find the time in flight, divide the distance by the groundspeed.

Example:	The time to fly 120 N. miles at a ground speed of 80

knots is 120 divided by 80 = 1.5 hrs (the 0.5 hrs x 60

= 30 min).

Answer: 1 hr 30 min.

d.	Distance

To find the distance flown in a given time, multiply groundspeed by

time.

Example:	The distance flown in 1 hr 45min at a groundspeed of

120 knots is 120 x 1.75 = 210 n. miles.

e.	Groundspeed

To find the groundspeed, divide the distance flown by the time.

Example:	An airplane flies 270 N. miles in 3 hrs. The

groundspeed is 270 divided by 3 = 90 knots.

CONFIRMATION STAGE 1

1.	How long would it take to fly 150 N. miles at a groundspeed of 100 kts?

Answer:	150 divided by 100 = 1.5 hrs

0.50 x 60 = 30 min

1 hr 30 min

2.	What is the distance flown in 1hr 15 min at a groundspeed of 80 kts.

Answer:	15 divided by 60 = 0.25

80 x 1.25 = 100 miles

3.	What was the groundspeed of an aircraft that flew 180 N miles in 1 hr and 		30 min?

Answer:	30 divided by 60 = 0.5

180 divided by 1.5 = 120 kts

STAGE 2 	NAVIGATION RULES

18

MINs

1.	The "One-in-Sixty" Rule:

OHP # 3

a.	An error in the track of one degree will cause an error in position of 			about one mile in a distance of 60 miles.

b.	A pilot on a cross-country flight who has drifted off the intended 			track can estimate the distance from the intended track in miles 			quite easily.  However, it would be more difficult to calculate the 			number of compass degrees the heading must be altered to correct			the error.

c.	Suppose an aircraft is two miles off its track after travelling 30

miles. The error in the track will be roughly 4º. Therefore the

correction to the compass heading will be 4º to correct the error.  			This will put the airplane on a track parallel to the required track but 		2 miles from it. Suppose the airplane is 60 miles from its 				destination. An additional 2º correction to heading will gradually 			close the track. Therefore, a total correction of 6º will bring the 			aircraft to its destination.

CONFIRMATION STAGE 2

1.	What is the “one in sixty” rule?

Answer:	An error in the track of one degree will cause an error in 				position of about one mile in a distance of 60 miles.

2.	After travelling 15 miles, an aircraft is ½ mile off course. The aircraft is 30 	miles from its destination.

a.	What is the error in track?

Answer:	2 degrees

b.	What correction will bring the aircraft to its destination?

Answer:	3 degrees

| 28 MINs | PERFORMANCE CHECK |
| --- | --- |

Test Details - 	Each cadet will have to identify units of distance and speed.

1.	Test knowledge with OHP # 4.

| 33 MINs | CONCLUSION |
| --- | --- |

SUMMARY:	A.	In this lesson you have learned about units of distance and speed.

RE-MOTIVATION:	A.	Comment on student performance. (Identify strengths and points that

require improvement)

B.	You will have an opportunity to use this knowledge during the last				navigation class, where you will plot a flight plan.

C.	Your next lesson is PO/EO 417.04a. In this class you will learn about plotting instruments.